Implementation of Fast Interconnects:

A)Fast Interconnects by Reducing the EPR Generation Time:

Proposed Idea:

A speed factor γ is introduced to model faster quantum interconnects.

The effective entanglement generation time is:

τeff=τeγ

where:

- τe= original entanglement generation time

- γ= interconnect speed factor

- τeff= effective entanglement generation time

What is Te according to IonQ paper?

1)Paper defines Te as the time taken to generate EPR pair relative to gate time.

2)It assumes EPR generation time is Te times greater than the gate time.

3)Te  is a factor not the real EPR time.

TEPR  = Te  * Tgate

Use: This assumption allows us to study the behaviour independent of hardware implementation because different hardware implementations have different gate times.

Real World EPR time:

| TITLE | YEAR | EPR TIME | FIDELITY |
| --- | --- | --- | --- |
| High-Rate, High-Fidelity Entanglement of Qubits Across an Elementary Quantum Network [E] https://doi.org/10.1103/PhysRevLett.124.110501 | 2020 | 5.5 ms | 94% |
| Fast Photon-Mediated Entanglement of Continuously Cooled Trapped Ions for Quantum Networking [E] https://journals.aps.org/prl/abstract/10.1103/PhysRevLett.133.090802 | 2024 | 4 ms | 94% |
| High-Fidelity Remote Entanglement of Trapped Atoms Mediated by Time-Bin Photonic Qubits [E] https://arxiv.org/pdf/2406.01761 | 2025 | 3 sec | >97% |
| Electron Juggling: Approaching the Atomic Physics Limit of the Attempt Rate in Trapped Ion Photonic Interconnects [T] https://arxiv.org/pdf/2510.27005 | 2025 | < 1 ms | >98% |

Range of Speed Factor gamma:

Since, the Real world EPR times are recorded between 5.5ms – 1ms, gamma can range from 1 to 5 or 10 atmost as per Present day Recorded times.

B)Bandwidth Model:

Instead of generating EPR pair fast, we focus on producing Multiple EPR pairs in a given time.

In modular and distributed quantum computing architectures, not all qubits are directly involved in computation. A subset of qubits, called communication qubits, is dedicated to interacting with optical or photonic interfaces used for remote entanglement generation.

Communication qubits are used in EPR pair generation. One communication qubit from each participating QPU is generally required to establish one EPR pair. Therefore, a communication-qubit pair acts as a single EPR-generation unit, capable of producing one Bell pair at a time.

So, we can produce multiple Bell Pairs parallely by increasing the number of communication qubits in a QPU.

Advantages:

- More Bell-pairs can be generated simultaneously.

- Waiting time will decrease.

Disadvantages:

- If we increase the number of qubits for communication, qubits for computation will be less and effects the computation.

So, there should be trade-off between the allocation.

Note: No papers found. SwitchQnet assumed the number of qubits for communication are 2 and stated the same result.

Other ways for making interconnects faster.

C)EPR Buffering:

In this method, EPR pairs are generated before-hand and used when required.[ ready-made].

Advantages:

- No need to wait for Bell pair

Disadvantage:

- EPR pairs stored for long time can result in Decoherence.

- Requires additional hardware

Source: https://arxiv.org/pdf/2208.06724 [CollComm ]

D)Multiple EPR attempts:

Instead of waiting for the outcome of one entanglement attempt before initiating the next, multiple entanglement-generation attempts are performed. This creates multiple opportunities for successful Bell-pair generation. Successful EPR pairs are used or stored for later use.

Advantages:

- Slightly better than Buffering method.

Disadvantages:

- Requires additional hardware.

Source: https://arxiv.org/pdf/2103.08269