Interconnect Speed in Bytes/sec

Quantum interconnect bandwidth ----- The speed at which the quantum information safely travel between isolated QPUs.

Conversion of EPR rate to Bytes/sec

During the quantum teleportation process, the sender’s measurement destroys the qubit and generates 2 classical bits of information (00, 01, 10, or 11).

These classical bits are used for the state reconstruction in other QPU.

Teleporting one qubit consumes one Bell pair and requires transmission of two classical bits.

1 Byte = 8 bits   => 1 Bell Pair = 0.25 Bytes.

Real World EPR time:

| TITLE | YEAR | EPR TIME | FIDELITY | n.o of EPR Pairs per sec | Bandwidth Bytes/sec |
| --- | --- | --- | --- | --- | --- |
| High-Rate, High-Fidelity Entanglement of Qubits Across an Elementary Quantum Network [E] https://doi.org/10.1103/PhysRevLett.124.110501 | 2020 | 5.5 ms | 94% | 184 | 45.5 |
| Fast Photon-Mediated Entanglement of Continuously Cooled Trapped Ions for Quantum Networking [E] https://journals.aps.org/prl/abstract/10.1103/PhysRevLett.133.090802 | 2024 | 4 ms | 94% | 250 | 62.5 |
| High-Fidelity Remote Entanglement of Trapped Atoms Mediated by Time-Bin Photonic Qubits [E] https://arxiv.org/pdf/2406.01761 | 2025 | 3 sec | >97% | 0.35 | 0.0625 |
| Electron Juggling: Approaching the Atomic Physics Limit of the Attempt Rate in Trapped Ion Photonic Interconnects [T] https://arxiv.org/pdf/2510.27005 | 2025 | < 1 ms | >98% | 1000 | 250 |
| High-Rate and High-Fidelity Modular Interconnects Between Neutral Atom Quantum Processors. | 2024 | 10*10-6 | 99.9% | 105 | 25000 |